
RS-422/485 Terminal Block Wiring and DIP Switch Settings

For USB To RS-422/485 Converter
Terminator (DIP Switches)
The purpose of the Terminator (DIP Switches) is for compensating signal attenuation in a long distance connection at RS-485/422.  To activate the signal compensation, please set the dip switch 1 & 2 to “ON” position.  To disable signal compensation, please set dip switch 1 & 2 to “OFF” position.
□RS-422 Pin Assignment
The pin assignment scheme for a 4-pin RS-422 is given below.




RS-422:  PIN 1: T+    PIN 2: T-   PIN 3: R+   PIN 4: R-
RS-485:  PIN 1: D+    PIN 2: D-
□RS-422 Wiring Diagram
Serial Device                USB To 422/485 Converter
    R-                              2  T-

    R+                             1  T+

    T-                              4  R-

    T+                             3  R+

□RS-485 Wiring Diagram
Serial Device               USB To 422/485 Converter
   D-                              2  D-  
   D+                              1  D+ 

















































1  2  3  4

















