Bluetooth (Class 1) To RS-232 Converter

It allows any device with an RS-232 port to communicate wirelessly.

No software installation is needed and no changes in the exiting system are required.    The Serial Port Adaptor is used in any application currently using an RS-232 cable.

It also can communicate with other Bluetooth device such as USB Dongle.

Key Features:

. Replace any existing RS-232 cable

. Full Bluetooth 2.0 support +EDR

. No external software needed.

. Easy to mount

. Easy to configure Baud Rate and master or slave role is selectable.

. Support baud rate Max 115.2 K bps.  Min 1200 bps.

. Plug-In-Design, 5 V DC external Power supply

. 300 feet (Enhance Antenna) CLASS 1

Specification:   Construction:

ABS cover transparent blue

Dimensions: 65 x 30 x 12 mm (W*D*H, not including Antenna)

Weight: 38 grams (Not including Power)

Interfaces:

Bluetooth: version 2.0 + EDR

Supported Bluetooth Profiles: Generic Access Profile , Serial Port Profile

Bluetooth radio: class 2 / class 1

Serial interface: RS-232 Female DSUB-9 connector (RX/TX crossed over)

Power connector: External adapter with DC Jack 3.5 x 1.3 mm

Power supply: DC5V 200mA, current consumption 80 mA

Plug-and Play environments

Operating temperature: +0 ~ +60°C

Storage temperature: -15 ~ +70°C

External ANT (optional): Reverse SMA connector